[image: image3.jpg]KEFID Kefid Machinery Co.,Ltd is a leading company specially
. . involved in the manufacture and sales of large-scale
Reach new heightwith | crushers and industrial mitl






Screw sand washing machine


 


Screw sand washing machine is mainly used for sand processing, electric pole factory, building work site and concrete dam, various sands classifying and dewatering. 

This sand washer adopts inclination of 15°,　under the water tub, three weir plates form the sediment pool, the screw head is immersed in this pool, the screw is driven by electric motor through reducer rotating continuously, the fresh water is feeding from porous plate at bottom of sediment pool, this machine have three functions of washing, dewatering, classifying. 
Working Principle

Washing: The sand is feeding into the sediment pool from the feeding box, acting by rotating screw blade, the sand is turning and rolling and grinding each other to eliminate impurity and water-air layer around sand to facilitate de-water. The fresh eater rising current from the porous plate carry foreign matter to water surface and overflow the weir plate, to finish washing processing 

De-watering: The sand with bigger density is sunken down to tub bottom, the screw blade push this sand upwards, when passing through upper tub of proper length above water level, the water is draining into sediment pool from another drainage ditch, the de-watered sand is out from the discharging opening. 

Classifying: Material classifying is realized by overflowing the non-needed fines, the controlling method is adjusting height of overflowing weir and adjusting speed of screw.
Technical Parameter:

	Models
	LSX-920
	2LSX-920
	LSX-1120
	2LSX-1120

	Diameter of screw (mm)
	920
	920
	1120
	1120

	Length of tub (mm)
	7585
	7585
	9750
	9750

	Feed Size (mm)
	≤10
	≤10
	≤10
	≤10

	Capacity (t/h)
	100
	200
	175
	350

	Speed of screw (r/min)
	21
	21
	17
	17

	Electric Motor Power(kW)
	11
	2x11
	18.5
	2x18.5

	Water Consumption (t/h)
	10-80
	20-160
	20-150
	40-300

	Dimension (mm)(L×B×H)
	8420×2180×3960
	8420×3970×3960
	10770×3950×4860
	10770×5260×4860

	Weight (t)
	6.2
	11.5
	10.9
	17.8


Note: Any change of Screw Sand Washing Machine technical data shall not be advised additionally.

[image: image4.jpg]




[image: image3.jpg][image: image4.jpg]