[image: image3.jpg]KEFID Kefid Machinery Co.,Ltd is a leading company specially
. . involved in the manufacture and sales of large-scale
Reach new heightwith | crushers and industrial mitl






Vibrating Feeder

 


Vibrating Feeder is a kind of linear direction feeding equipment. It features smooth vibrating, reliable operation, long service life and being suitable for feeding. They are widely used in mining, building-material, silicate and chemical industry in the crushing and screening plants.

Main Features:

1. stable and long span life

2. easy to adjust

3. reliable operation.

4. easy maintenance.
Vibrating Feeder Operating principle :
Vibrating feeder consists of vibrating frame, spring, vibrator, motor vibrating device and motor. The vibrator is made of two fixed eccentric shafts whose gears are jogged. When installation, the two gears must be jogged according to the sign. Through drive of the motor, the two eccentric shafts start rotating to produce a linear huge power which forces the feeder to vibrate. Through the vibration, The materials will slip and sling on the funnel, moving forward, when the materials pass the screening part, smaller parts will fall down, avoiding the further crushing, so screening purpose can be reached.

Technical Parameter:

Mm to inch conversion: 25.4millimeters=1 inch

 

	Model
	Max. feeding
size(mm)
	Handling
capacity(t/h)
	Power (kW)
	Dip angle(º)
	Weight(kg)

	GZG70-4
	200
	120-170
	2X0.5
	~10
	389

	GZG100-4
	300
	270-380
	2X1.1
	~10
	762

	GZD240X70
	400
	30-80
	2x1.5
	~10
	3000

	GZD300×85
	450
	80-120
	2x3.7
	~9
	3900

	ZSW380×96
	500
	120-210
	11
	0
	4200

	ZSW420×110
	650
	180-400
	15
	0
	4800

	ZSW490×110
	650
	250-480
	15
	0
	5300

	ZSW490×130
	750
	300-550
	22
	0
	6700

	ZSW600×130
	750
	400-800
	30
	0
	7800

	ZSW600×150
	800
	500-850
	30
	0
	8800


 

Note: Any change of Vibrating Feeder technical data shall not be advised additionally.

[image: image4.jpg]




[image: image3.jpg][image: image4.jpg]